## RSSI Measurements

In order to make measurements using the iwlist command as implemented in this CLS context the user has to do the following:

- cd iwlist

- make clean

- make all

- iwconfig <interface> mode managed

- ./iwlist <interface> <file>

  - interface : eth1, ath0, wlan0 depends on the wireless card used and can be configured by typing iwconfig in command line.

  - file : is the name or path of the file that the obtained measurements are going to be saved.

For the execution of the iwlist, root permission may be  needed. So execute the last command as:

sudo ./iwlist <interface> <file> and enter the super user password.

The user must kill the program (Ctrl+C) in order to stop.

After the program is killed a file is created with the name <file> composed by three columns. The first one illustrates the timestamp of the system when the measurement was taken, the second one the AP’s MAC address and the third one the signal strength (SS) measurement collected. The measurements are sorted per AP.